| Name: Enrolment No: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # Odd Semester Mid-Term Examination, September 2025 # FOSTA, School of Computer Science & Engineering # Artificial Intelligence & Machine Learning Department B.Tech. - Artificial Intelligence & Machine Learning |  |  |  |  |  |  |  |
| # Course Code: AIM3144 |  | # Course: HIGH PERFORMANCE COMPUTING |  |  |  | # Semester: 5th |  |
| Time: 1.5 hrs. Max. Marks: 30 |  |  |  |  |  |  |  |
| Instructions: All questions are compulsory. Missing data, if any, may be assumed suitably. Calculator is allowed. |  |  |  |  |  |  |  |
| SECTION A (Memory Based Questions) |  |  |  |  |  |  |  |
| S.No. |  |  |  |  | Marks |  | CO |
| A1 | Explain the MIPS1 instruction set architecture. |  |  |  | Max Marks 6 |  | CO1 |
| A2 | Compare between LB (load byte) and LBU (load byte unsigned). Explain with examples. |  |  |  |  |  | CO1 |
| A3 | i) Which MIPS instruction is typically used to return control to the caller function? a) J b) JR $ra c) JAL d) BNE |  |  | ii) Which of the following is a branch instruction in MIPS? a) LW b) SW c) BEQ d) JAL |  |  | CO2 |
| SECTION B (Concept Based Questions) |  |  |  |  |  |  |  |
| B1 | Consider the following MIPS instructions that load and store data in memory. Assume the system uses Little-endian byte ordering. LBU R1, 0(R2) LB R3, 1(R2) Assume register R2 = 0x5000 and the memory content starting from 0x5000 is: Address; Value (Hex). 0x5000; 0x7F. 0x5001; 0xA0. a) What value will be loaded into R1 after the first instruction (LBU R1, 0(R2)) executes? b) What value will be loaded into R3 after the second instruction (LB R3, 1(R2)) executes? | Max Marks 16 |  | CO2 |
| B2 | Given the following MIPS instructions: ADD R1, R2, R3 # Instruction A LW R4, -12(R2) # Instruction B ADDI R5, R4, -20 # Instruction C SW R6, 0x1C(R3) # Instruction D a) Identify the addressing mode used in each instruction. b) If R2 = 0x10010020, what is the effective address for Instruction B? c) If R3 = 0x10010000, what is the effective address for Instruction D? d) After executing Instruction C, what will be the value in R5 if R4 = 0x00000015? |  |  |  |  |  | CO3 |
| B3 | Compare between General Purpose Registers and Special Purpose Registers. Explain why general-purpose registers are used in a processor. |  |  |  |  |  | CO1 |
| B4 | Consider a 32-bit IEEE 754 floating-point number represented as: 0 10000001 00000000000000000000000 a) Identify the sign bit, exponent, and mantissa. b) Calculate the value of the floating-point number in decimal. |  |  |  |  |  | CO2 |
| SECTION-C (Analytical Based Questions) |  |  |  |  |  |  |  |
| C1 | Consider the following instructions: MULT R5, R6 # Instruction 1 MFLO R7 # Instruction 2 MFHI R8 # Instruction 3 If the values of R5 and R6 are: R5 = 0xFFFF0001 (or -65535 in signed decimal) R6 = 0x00000002 (2 in decimal) a) Evaluate the signed multiplication of R5 and R6? Show the 64-bit result in hexadecimal. b) Evaluate the values in the HI and LO registers after Instruction 1? c) Evaluate the values that will be stored in R7 and R8 after Instructions 2 and 3 execute? d) Why does MIPS use HI and LO registers for storing multiplication results instead of just one general-purpose register? |  |  |  | Max Marks 8 |  | CO2 |

In MPIS-1, coherence is maintained using:
A. Directory protocol
B. Bus snooping
C. Token-based approach
D. Distributed directories