# Solutions - AIM3144 MTE (High Performance Computing)

## Section A: Memory Based Questions

### A1. Explain the MIPS1 instruction set architecture.

The MIPS-I instruction set architecture (ISA) is a Reduced Instruction Set Computing (RISC) architecture developed by MIPS Computer Systems. Key features include:
• 32-bit fixed-length instructions for simplicity and faster decoding.
• Load/Store architecture: memory access only through load and store instructions.
• 32 general-purpose registers (GPRs), each 32 bits wide.
• Three instruction formats: R-type (register), I-type (immediate), J-type (jump).
• Support for arithmetic, logical, control flow, and memory instructions.
• Program counter (PC) to hold the address of the current instruction.

MIPS-I follows a simple and efficient design philosophy making it suitable for pipelining and parallel execution.

### A2. Compare between LB (load byte) and LBU (load byte unsigned). Explain with examples.

• LB (Load Byte): Loads a signed byte (8 bits) from memory into a register and sign-extends it to 32 bits.
• LBU (Load Byte Unsigned): Loads an unsigned byte (8 bits) and zero-extends it to 32 bits.

Example:
Memory[0x1000] = 0xFE (in hex, signed = -2, unsigned = 254)
  - LB R1, 0(R2) → if R2=0x1000, then R1 = 0xFFFFFFFE (-2 signed).
  - LBU R1, 0(R2) → R1 = 0x000000FE (254 unsigned).

### A3. Multiple Choice Questions

i) The MIPS instruction typically used to return control to the caller function is **JR $ra**.
ii) The branch instruction in MIPS is **BEQ**.

## Section B: Concept Based Questions

### B1. MIPS Load/Store with Little-endian

Given:
R2 = 0x5000
Memory[0x5000] = 0x7F, Memory[0x5001] = 0xA0

a) LBU R1, 0(R2): Loads unsigned byte from 0x5000 → 0x7F. Zero-extended → R1 = 0x0000007F.
b) LB R3, 1(R2): Loads signed byte from 0x5001 → 0xA0. Since MSB=1, sign-extended → R3 = 0xFFFFFFA0 (-96 in decimal).

### B2. Addressing Modes and Effective Addresses

a) Instruction addressing modes:
  • ADD R1, R2, R3 → Register addressing.
  • LW R4, -12(R2) → Base/displacement addressing.
  • ADDI R5, R4, -20 → Immediate addressing.
  • SW R6, 0x1C(R3) → Base/displacement addressing.

b) If R2 = 0x10010020, effective address for LW R4, -12(R2):
  EA = 0x10010020 + (-12) = 0x10010014.

c) If R3 = 0x10010000, effective address for SW R6, 0x1C(R3):
  EA = 0x10010000 + 0x1C = 0x1001001C.

d) If R4 = 0x00000015, ADDI R5, R4, -20:
  R5 = 0x00000015 + (-20) = 1=0x00000001

### B3. General Purpose vs Special Purpose Registers

• General Purpose Registers (GPRs): Used for arithmetic, logic, and data movement. Flexible and can be used by compilers to optimize code.
• Special Purpose Registers (SPRs): Used for specific functions such as PC (Program Counter), HI, LO, and status registers.

General-purpose registers are widely used because they allow fast access to frequently used data and reduce the need for repeated memory accesses, improving CPU performance.

### B4. IEEE 754 Floating Point Representation

Given: 0 10000001 00000000000000000000000

• Sign bit = 0 → Positive number.
• Exponent = 10000001 (binary) = 129. Bias = 127 → Actual exponent = 129 - 127 = 2.
• Mantissa = 1.0 (since fraction is all zeros).

Value = 1.0 × 2^2 = 4.0 (decimal).

## Section C: Analytical Based Questions

### C1. MIPS Multiplication and HI/LO Registers

Given:
R5 = 0xFFFF0001 (signed = -65535)
R6 = 0x00000002 (signed = 2)

a) Signed multiplication: -65535 × 2 = -131070.
In 64-bit hex: 0xFFFFFFFFFFFE0002.

b) HI and LO registers:
  • LO = lower 32 bits = 0xFFFE0002.
  • HI = upper 32 bits = 0xFFFFFFFF.

c) After MFLO R7 → R7 = LO = 0xFFFE0002.
   After MFHI R8 → R8 = HI = 0xFFFFFFFF.

d) MIPS uses HI and LO registers because multiplication produces a 64-bit result, which cannot be stored in a single 32-bit general-purpose register.