# High-Performance Computing - Question Bank

- Match the following instructions with addressing modes:
ADD R3, R4, R5; LW R6, 8(R2); ADDI R7, R1, 12; SW R8, 16(R9)

- Match instructions with correct addressing types:
SUB R1, R2, R3; LW R4, 20(R5); ADDI R6, R7, 9; SW R2, 24(R8)

- Identify the addressing mode used in: LW R5, 32(R6)

- Identify the addressing mode in: ADDI R2, R3, 15

- Which of the following is a branch instruction?
a) LW b) SW c) BEQ d) ADD

- Which instruction is used for unconditional jump?
a) BNE b) BEQ c) J d) LW

- Which instruction returns control to caller?
a) J b) JR $ra c) ADD d) SW

- Which instruction saves return address?
a) J b) JAL c) JR d) BNE

- Which register holds return address?
a) $t0 b) $ra c) $sp d) $s1

- Define parallel computing systems in one line.

- List types of parallel computers.

- Explain SIMD architecture in one line.

- Define MIMD system.

- What is SISD architecture?

- Differentiate shared vs distributed memory.

- Given instructions:

SUB R1, R2, R3

LW  R4, 16(R1)

SW  R5, 24(R1)

JR  R6

a) Compute cycles for SUB instruction
b) Compute cycles for SW instruction
c) Compute cycles for JR instruction.

Explain pipeline stages.

- Given:

R2 = 0x9000

Memory:

0x9000 → 0x7A

0x9001 → 0xC5

Instructions:

LBU R1, 0(R2)

LB  R3, 1(R2)

Find the values loaded into R1 and R3.

- Given the following MIPS instructions. Assume the system uses small endian byte ordering.

R4 = 0xA000

Memory:

0xA000 → 0xFF

0xA001 → 0x20

Instructions:

LB  R5, 0(R4)

LBU R6, 1(R4)

Find values in R5 and R6.

19) Consider the following MIPS instructions. Assume the system uses big endian byte ordering.

LBU R2, 0(R3)

LB  R4, 1(R3)

Assume:

R3 = 0x6000

Memory:

| Address | Value (Hex) |
| --- | --- |
| 0x6000 | 0x45 |
| 0x6001 | 0xF2 |

What value is loaded into R2 after LBU?

20. Given the following MIPS instructions:

ADD R1, R2, R3 Instruction 1; LW R4, 8(R1)              Instruction 2

SW R5, 12(R1) Instruction 3; JR R6                       Instruction 4

a) Calculate the total number of cycles required to execute the SW (Store Word)

b) Calculate the total number of cycles required to execute the JR (Jump Register) instruction? Explain each stage.

21. Which instruction is used to return from a function call?
a) J b) JR $ra c) BEQ d) ADD

22. The following MIPS instructions use different addressing modes. Identify the addressing mode used in each instruction and match them correctly.

- SUB R2, R3, R4

- LW R5, 12(R6)

- ADDI R7, R8, 10

- SW R9, 20(R1)

- Immediate Addressing
B. Register Addressing
C. Base/Displacement Addressing

D.Base/Displacement Addressing

Match the instructions with the correct addressing mode.