| Name: Enrolment No: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # Odd Semester End Term Examination, Nov 2025 # FoSTA, School of Computer Science & Engineering # Department of Artificial Intelligence & Machine Learning B.Tech. CSE (AIML) |  |  |  |  |  |  |
| # Course Code: AIM3144 |  | # Course: High Performance Computing |  | # Semester: V |  |  |
| Time: 03 hrs. Max. Marks: 80 |  |  |  |  |  |  |
| Instructions: All questions are compulsory. Missing data, if any, may be assumed suitably. Calculator is allowed. |  |  |  |  |  |  |
| SECTION A (Memory Based Questions) |  |  |  |  |  |  |
| S.No. |  |  |  |  | Mark | CO |
| Q A1 | State the advantages of using a load/store architecture such as MIPS. |  |  |  | 2*5= Max Marks 10 | CO1 |
| Q A2 | True or false: a) MIPS avoids complex instructions like indirect addressing to maintain one instruction per cycle. b) In base–displacement addressing, the base register is always the program counter. |  |  |  |  | CO1 |
| Q A3 | Describe the three instruction formats in MIPS-I (R, I, J). Provide one example of each. |  |  |  |  | CO2 |
| Q A4 | A processor uses pipelining to execute 12 instructions in 16 cycles. How many stages are there in the pipeline? |  |  |  |  | CO4 |
| Q A5 | Compare pipelined vs non-pipelined execution using a space-time diagram. |  |  |  |  | CO4 |
| SECTION B (Concept Based Questions) |  |  |  |  |  |  |
| Q B1 | Explain why the MIPS instruction LB (Load Byte) performs sign extension, while LBU (Load Byte Unsigned) performs zero extension. Describe how this difference affects the interpretation of the loaded data. |  |  |  | 5*6=Max Marks 30 | CO1 |
| Q B2 | Consider the following MIPS instructions: SUB R7, R8, R9 # Instruction 1 LW R10, 12(R8) # Instruction 2 ADDI R11, R10, 4 # Instruction 3 SW R12, 20(R9) # Instruction 4 a) Identify the addressing modes used in Instruction 1, Instruction 2, Instruction 3, and Instruction 4. b) If R8 = 0x20000040, compute the effective address for Instruction 2. c) If R9 = 0x20000020, compute the effective address for Instruction 4. d) If the value in R10 = 0x30 before Instruction 3 executes, determine the value stored in R11 after Instruction 3 completes. |  |  |  |  | CO2 |
| Q B3 | A non-pipelined processor completes one task in 60 ns, while an upgraded 5-stage pipelined version runs with a 12 ns clock cycle. Using your understanding of pipelined versus non-pipelined execution, apply the pipeline execution model to determine the approximate speedup when processing 400 tasks on this processor. |  |  |  |  | CO4 |
| Q B4 | Consider the following scenario in an MPI-based system: - Process P0 wants to broadcast a message to all other processes. a) What is the MPI function used to broadcast a message from one process to all others? b) What is the difference between point-to-point communication and collective communication in MPI? Provide an example for each. |  |  |  |  | CO5 |
| Q B5 | Given the following MIPS instructions: SUB R7, R8, R9 # Instruction 1 LW R10, 16(R7) # Instruction 2 SW R11, 20(R7) # Instruction 3 J 1000 # Instruction 4 a) Calculate the total cycles needed for the SUB instruction and explain the function of each stage. b) Determine the cycles for the LW instruction and describe the role of each stage, including memory access. c) Compute the cycles for the SW instruction, highlighting the memory write stage. d) Compute the cycles for the J (Jump) instruction and explain the function of each stage. |  |  |  |  | CO3 |
| SECTION-C (Analytical Based Questions) |  |  |  |  |  |  |
| Q C1 | In a shared-memory multiprocessor system, two processors P1 and P2 maintain private caches. A shared variable X is cached by both processors. The initial value of X in main memory is 50. Processor P1 updates its cached copy to 120, while P2 continues to use its old cached copy without issuing any coherence action. The system uses a snooping-based cache coherence mechanism. a) Analyze the type of cache incoherence that occurs in this scenario and explain why it arises. b) The system designer considers implementing the Write-Once protocol. Evaluate how this protocol would handle the write performed by P1, the subsequent read by P2, and the transitions between cache states. c) If P2 temporarily disables its snooping mechanism to reduce power consumption, evaluate the consequences on both correctness and system performance. Support your answer with possible system outcomes. |  |  |  | 2*10=20 Marks | CO5 |
| Q C2 | Analyze the performance impact of introducing pipelining into a processor architecture. A non-pipelined processor executes each instruction in 8 cycles and operates at a 2 GHz clock frequency. A redesigned version incorporates an 8-stage pipeline, but due to latch overhead, the clock frequency decreases to 1.6 GHz. Evaluate how the architectural modification affects system performance by computing the speedup achieved by the pipelined processor compared to the non-pipelined version. |  |  |  |  | CO4 |
| SECTION-D (Case Study/Application based Question) |  |  |  |  |  |  |
| Q D1 | Design and compute the total execution time for a custom 7-stage pipelined processor. The proposed processor has stage delays of 25 ns, 27 ns, 30 ns, 22 ns, 24 ns, 26 ns, and 28 ns, and each interstage register adds an additional 1 ns delay. Assuming ideal pipeline behavior with no stalls or hazards, construct the timing model and determine the total time required to process 10,000 tasks using this pipeline. |  |  |  | 2*10= 20 Marks | CO4 |
| Q D2 | Using the 32-bit floating-point pattern: 1 01111101 10000000000000000000000. Design a step-by-step explanation that demonstrates how a 32-bit IEEE-754 single-precision floating-point number is interpreted. Break down the bit pattern into the sign, exponent, and mantissa fields, explain the meaning of each field, and show how these components are combined mathematically to produce the final decimal value. Structure your explanation so that someone unfamiliar with the conversion process can understand how each part contributes to the resulting number. |  |  |  |  | CO2 |